How PL-2303H/X driver works with Modem on Linux RedHat 8.0

- Copying pl2303.c to the /usr/src/linux2.4/drivers/usb/serial.

- Change directory to /usr/src/linux2.4

- copy /usr/scr/linux2.4/configs/kernel-2.4.18-i686.config to /usr/src/linux2.4 as .config

- Type “make menuconfig” on /usr/src/linux2.4 directory.

- Choose “Loadable module Support --” to uncheck “Set version information……” item and then leave this sub menu. (Appendix A.)

- Choose “General setup --” to selects “[*] Kernel .config support” and leave this sub menu.

- Choose “USB support --” “USB Serial Converter support --” to uncheck all the driver module expect “USB Generic Serial Driver” and “USB Prolific 2303 Single …….” and then leave this sub menu.

- Finally to leave this menuconfig and save it.

- To modify the extension name of Makefile as you want to show up on GRUB menu.

- Type “make clean; make dep; make bzImage; make modules; make modules_install; make install” and click “Entry”.

- Wait for kernel compiled, and the reboot the system, and then to choose kernel name that you modified on GRUB menu to boot you own system up.

- After system ready, type “uname –all” to check the kernel name is your defined (for example: linux-2.4.18-14philip), if it is original Linux kernel name, you have some thing wrong during building kernel, please go back to the step 3 to try again.

- If it is successfully, go /lib/modules/2.4.18-14philip/kernel/drivers/usb/serial to check pl2303.o has exist this folder, if it is not, please go back to step 10 to try again.

- Plug in PL-2303X to the USB port, and type “dmesg” on consol, the last message will identify the PL-2303X attached on ttyUSB0.

- Type minicom –s xxxxxx to create a script file for PL-2303X modem.

- When you first time to runs minicom, it will stay on consol mode, they “CTRL-A and then Z” to pop menu up, and then choose “O” to configure modem.

- On “configuration” menu, choose “Serial port setup” to set A—Serial Device: as /dev/ttyUSB0, and then set E – Bps/Par/Bits: as 57600 8N1 and then leave.

- On the same menu, choose “Save setup as xxxxxx” and then choose “exit” to initialization modem and minicom.

- If your setup is correctly, now the state would be modem ready.

- Type “CTRL-A and then D” to open dial up setting menu to fill in necessary information for dial, and then try to dial up modem to connect to ISP.

- If it is successfully, now the PL-2303X is ready for use.

- Close minicom to return X windows.

- Choose StartSystem Toolsinternet Configuration Wizard to select “Device Type” as Modem connection and click Forward, fill in dialup information on next dialog box, and the click Apply.

- Selects Hardware page on next dialog box to set Modem Device: /dev/ttyUSB0, Baud Rate: 115200, Flow Control: Hardware (CRTSCTS), and then click OK.

- Go back to Devices page to choose a Profile, and then click Activate button to dial up to ISP.

- If it is successfully, now you can join to internet by the browser.

Appendix A. about how to building a custom kernel, please refer the www.redhat.com/docs/manuals/linux